# Self-purging inflation valve core with a deflectable seat ring — Prosecution Assessment

| Likelihood of grant | 6 / 10 | Claim 1 is not disclosed by any saved reference, but the only built embodiment is described with a mechanism that cannot work as written and its layout must come from the inventor before filing; scored on the application as reviewed in round 1, so fixes made in the later fix pass are not re-scored, and the score rests only on the 17 references we found and saved. |
| --- | --- | --- |

## Deficiencies and prior art the application will face

Status key: open, routed to fix pass = upheld and sent to the fix pass running in parallel (not re-reviewed); open = upheld or unclear and not routed; prior art = a saved reference. Rows marked (unclear) turn on facts only the inventor or counsel can supply.

| # | Deficiency or prior-art reference | Type | Claims affected | Severity | Source | How to respond | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | The description never explains how the purge shoulder reaches the seat ring. The poppet opens towards the chamber ([0067]), so a shoulder "on the chamber side of the sealing face" ([0059], claim 3) moves away from the ring as it opens, and no ring surface for it to press on is described. | Enablement | 3, 4, 22 | high | F-32-20 | Get the axial order of the parts (internal shoulder, sealing edge, convolutions, purge shoulder) and the ring face contacted from the inventor's sketch or prototype; add one paragraph; correct [0138] and claim 3 if reversed. This cannot be added after filing. | addressed in fix pass (not re-reviewed) |
| 2 | The same layout gap seen from the ring bore: the 2.3 mm bore admits only the poppet stem ([0110], [0111]) and [0138] puts the sealing edge at the supply-side end, so the preferred form of claims 3 and 4 cannot be built from the words, and there are no drawings to help. | Enablement | 3, 4, 8 | high | F-gate14-2 | As row 1: one layout paragraph from the inventor's FIG. 1 sketch, drawn; conform [0042], [0059], [0134] and claim 3 if the shoulder sits elsewhere. | addressed in fix pass (not re-reviewed) |
| 3 | For the illustrated core, no text says which ring surface takes the poppet's load, or how purge air in the outer clearance gets past the press-fit band at the internal shoulder to the chamber side ([0057], [0075], [0078], [0081]). The figure plan itself calls that arrangement a drafting assumption. | Enablement | 1, 4, 21, 30, 31, 58, 79 | high | F-28-3 | Inventor to state the loaded surface and the purge exit route; add them to [0059], [0074], [0078], [0081]; reconcile [0101], [0127] and [0138]. No claim narrowing. | addressed in fix pass (not re-reviewed) |
| 4 | Same gap, as it bears on claim 1's through-flow for the FIG. 1 core. Claim 1 itself survives on the coherent inner-surface arrangement of [0127], but claims 4, 21 and 30 read directly on the unexplained embodiment. | Enablement | 1, 4, 21, 30, 58, 79 | high | F-36-1 | As row 3: state the axial arrangement and the purge route (past the free end, through purge ports 262 or flutes 362) from the inventor's sketches. | addressed in fix pass (not re-reviewed) |
| 5 | The built core is said to open its purge gap by deflecting outward ([0041], [0075]), yet the gap lies between the ring's outer surface and the bore ([0061]). Moving outward closes that clearance, as [0125] itself says. The wiper ring has the same contradiction ([0115], [0185]). | Enablement | 9, 29, 30, 34, 72 | high | F-28-2 | Inventor to state the real movement of prototypes P-1 to P-3; rewrite [0041], [0061], [0075], [0076], [0115], [0185], the flutes of [0132] and the FIG. 1, 3, 6 and 10B instructions to it. No claim narrowing. | addressed in fix pass (not re-reviewed) |
| 6 | Same contradiction, as it bears on Example 1 and on claims 29 with 30 and claim 72. The only tested embodiment is described with a mechanism that cannot work as written, which weakens its value as evidence. | Enablement | 1, 9, 29, 30, 72 | high | F-30-6 | As row 5. If the movement is inward or ovalising, amend "outward" in the illustrated embodiment; claim 1 is not narrowed. | addressed in fix pass (not re-reviewed) |
| 7 | Same contradiction, plus the "free" part of the ring's outer surface ([0057], [0075], [0125]) reads as a clearance at rest, against [0061] "the purge gap 60 is closed". | Enablement | 9, 29, 30, 72 | high | F-gate14-4 | As row 5; use the wording of [0125] if the gap opens by inward movement; state one at-rest condition. | addressed in fix pass (not re-reviewed) |
| 8 | (unclear) The test does not show that air pressure, rather than the pump head pressing on the poppet, caused the over-travel. The snap was heard only when the pump was removed ([0181]), and travel and pressure were not measured ([0179], [0188]). If the head pressed the poppet, the tested product falls outside claims 1, 58 and 79. | Enablement | 1, 58, 79, 83, 84, 87 | high | F-32-7 | Ask the inventor for the pump-head type and whether it touches the poppet tip, whether a snap occurred between strokes, and any pressure trace; correct [0065], [0090], [0181], [0202] to the answer. | open |
| 9 | The dimension and pressure ranges are stated to belong to the superelastic ring with a convoluted wall ([0110], [0113], [0116], [0168], [0184]) but are claimed for any material and wall, including a PEEK ring (claims 76–77) and a ring of no stated material (claim 67). | Support | 13, 16, 17, 35, 41–44, 57, 67, 69, 70, 76, 77, 82 | high | F-28-1 | Ask the inventor whether the ranges hold for PEEK, resilient metal and other walls. If yes, say so and delete the self-limiting sentences; if not, add "superelastic alloy" to claim 67 and remove the ranges from claims 76–77 (narrowing: counsel decides). | addressed in fix pass (not re-reviewed) |
| 10 | No drawings exist. The package holds only a figure plan, which also contains internal working notes that must never be filed. Filing without drawings risks re-dating or loss of the figure references at the EPO (Rule 56) and a drawings objection in the US (35 U.S.C. 113). | Formal | all | high | F-36-2 | Commission FIGS. 1–15 once rows 1–7 are answered; add a Brief Description of the Drawings; strip all working notes from the instructions. | addressed in fix pass (not re-reviewed) |
| 11 | No abstract, no brief description of the drawings, and no technical field, background or summary; [0183] points to a background that does not exist. | Formal | — | high | F-gate14-1 | Supply the missing sections (abstract and list of figures at assembly); re-point [0183] to the definitions at [0045] and [0050]. | addressed in fix pass (not re-reviewed) |
| 12 | Lack of unity: claims 67 (seat ring) and 90 (making it) share with claim 1 only a ring that deflects radially and recovers, which US 9982818 B2 already shows in wave form. There are also three product independents in one category (1, 58, 67), which needs a Rule 43(2) justification at the EPO. | Formal | 1, 58, 67, 79, 90 | high | F-30-7 | Tie claims 67 and 90 to the purge function (superelastic alloy, or the matched kit of [0163]), or divide them out. Expect a US restriction requirement. | addressed in fix pass (not re-reviewed) |
| 13 | Claim 67 (the seat ring alone) is a known wave-section ring (US 9982818 B2) made of superelastic NiTi (US 6393765 B1) and sized for a Presta bore. It recites no poppet, over-travel or purge gap. It is not anticipated, but the obviousness combination is credible. | Inventive step | 67–72, 76–78 | high | F-32-1 | Narrow: add superelastic alloy with Af below 5 °C and a continuous sealing edge, or recast as the matched kit of [0163]. | addressed in fix pass (not re-reviewed) |
| 14 | Claim 90 (making the ring) is the standard NiTi tube route: laser cut, shape-set in a fixture, quench, electropolish ([0169]–[0173]). It states no process effect, so it stands or falls with the ring. No saved reference documents this route; an examiner would cite general NiTi processing art. | Inventive step | 90, 92, 94–98 | high | F-32-2 | Tie claim 90 to the ring features that survive row 13, or to making the claim 1 valve core with the press-in step of claim 100. | addressed in fix pass (not re-reviewed) |
| 15 | Every independent claim requires the purge to happen because pressure pushes the poppet into over-travel. The description itself discloses three other routes a competitor can use freely: over-travel by a depressor or by hand ([0088]), a separate purge actuator ([0145]), and Schrader cores pressed by the pump head ([0159]). In the US, matter disclosed but not claimed risks being dedicated to the public. | Design-around | 1, 46, 57, 58, 79 | high | F-32-4 | Define the purge state by poppet position beyond the normal stroke, with the pressure trigger and mechanical depression as dependent alternatives; or add an independent claim with a pressure-responsive purge actuator. | addressed in fix pass (not re-reviewed) |
| 16 | Same escape for method claim 79, which requires "raising the pressure differential", and for deflation-only purging ([0089]). Against claims 1 and 58 a defendant uses [0080] and the word "additionally" in claim 46 to argue that a pressure-driven purge is required. | Design-around | 1, 46, 58, 79, 88 | high | F-33-4 | As row 15: a position-defined purge state, with pressure, depressor and deflation as dependents ([0033], [0088], [0089]). | addressed in fix pass (not re-reviewed) |
| 17 | The "normal-open" state requires the ring to be "in its at-rest form", so a design in which the poppet lightly touches the ring before the threshold escapes. Claim 40 (threshold set by ring stiffness, [0144]) also conflicts with claim 1, because there the ring is loaded below the threshold. | Design-around | 1, 40, 58, 79 | high | F-33-2 | Delete "the deflectable ring element is in its at-rest form and" from the normal-open state (broadening, supported by [0064], [0124]); amend [0140] and [0144] only if the inventor confirms that sub-threshold contact was conceived. | addressed in fix pass (not re-reviewed) |
| 18 | The wipe-only version ([0087]: ring sealed at its edge, cleaning by the snap-back wipe alone) is outside every independent claim. The test found that the wiped surfaces were the clean ones ([0190]). | Design-around | 1, 58, 67, 79 | high | F-gate14-5 | Add an independent core claim to over-travel deflection plus a recovery wipe across the poppet, without the purge gap ([0083], [0087], [0146]); counsel weighs unity. | addressed in fix pass (not re-reviewed) |
| 19 | (unclear) Every product claim names a return spring. A poppet closed by chamber pressure alone, with the threshold set by ring stiffness, would escape, but no such embodiment is disclosed ([0043], [0144]). | Design-around | 1, 58, 79 | high | F-33-5 | Ask the inventor whether a spring-less poppet was conceived and works; if so, disclose it before filing. | open |
| 20 | The only evidence of cleaning is on the wiped surfaces; film remained in the purge-gap region ([0182], [0190]). The effect of the purge path, the feature that distinguishes claim 1, is therefore unproven. | Inventive step | 1, 58, 79, 81 | medium | F-30-3 | Add any observation or data that separates purge from wipe; explain the purge effect in [0192]; keep claims 25–26 as fallbacks. | open |
| 21 | With the purge effect unproven, an examiner can treat claim 1 as an alternative second flow path and combine two-threshold valves (EP 1137892 A1, US 8043186 B2) with a deflecting NiTi ring (US 9982818 B2). Motivation is thin. | Inventive step | 1, 58, 79 | medium | F-32-10 | Add data isolating the air purge before filing or within the priority year; state the effect of the separate path; keep claims 25 and 47 as fallbacks. | open |
| 22 | Claim 1 covers every material, geometry and gap location, but only one NiTi species was tested, and purge and wipe were not separated. This invites a breadth attack (EPO plausibility; UK breadth-of-claim insufficiency). | Enablement | 1, 58, 79 | medium | F-30-11 | Obtain purge-versus-wipe data and a PEEK build if possible; make sure claims 11, 25 and 47 each stand as a fallback position. | open |
| 23 | The austenite finish temperature is claimed with no measurement method, and the value depends on the method used. | Enablement | 48, 74, 95 | medium | F-30-9 | Add the test method and standard actually used for the prototype rings. | open |
| 24 | Claim 90 says the convolution may be "defined by the laser cutting", but cutting a tube does not move the wall radially, and Example 1 never says which step formed the convolution ([0169]). | Enablement | 90, 93 | medium | F-32-15 | Record which step formed the convolution; delete "by the laser cutting" or explain it. | open |
| 25 | The worked-example dimensions do not visibly fit together: a 0.12 mm convolution on a 2.7 mm ring needs bore relief or a stem size that is never stated ([0111]). | Enablement | 13, 16, 69, 78 | medium | F-32-21 | State the direction of the convolutions, any bore relief, and the poppet stem diameter. | open |
| 26 | The FIG. 1 instruction draws a clearance at rest, handing an opponent a picture of the purge gap open when claim 1 says it is closed. | Support | 1, 58, 79 | medium | F-28-9 | Draw the free portion in contact at rest, or disclose what closes a running clearance to flow. | open |
| 27 | For the inner-surface gap of [0127], the main passage route is never described and "no significant flow" has no criterion. | Support | 1, 31 | medium | F-28-10 | Describe the main-passage route and replace "no significant flow" with a statable criterion. | open |
| 28 | The "any one of claims" dependencies create combinations the description contradicts, e.g. claim 47 on claim 24, claim 34 on claim 29, and claim 99 on the PEEK ring claims. | Support | 10, 13, 16, 17, 23, 24, 29, 34, 35, 41–43, 47–50, 76, 77, 99 | medium | F-28-11 | Prune the dependencies as listed in the finding; no supported scope is lost. | open |
| 29 | (unclear) The band-limited press fit of claim 21 is labelled in the package as a drafting assumption, not an inventor fact. | Support | 21 | medium | F-28-12 | Inventor to confirm; if not confirmed, delete claim 21 and the matching passages. | open |
| 30 | With a single-rate spring and a stiffness-set threshold (claim 40), the ring is loaded before the threshold ([0144]), so it is not "at rest" in the normal-open state. For polymer rings the threshold is "less distinct". | Support | 1, 40, 51–53 | medium | F-28-13 | Add inventor-confirmed wording that sub-threshold loading leaves the gap closed, or restrict claim 40 to the superelastic ring. | open |
| 31 | Fallbacks that pick one option from each of several lists rest only on the blanket statement in [0165], which the EPO and UK IPO rarely accept as a basis. | Support | 1, 58, 67, 79 | medium | F-29-5 | Add a short "preferred combinations" section on the model of [0167]. | open |
| 32 | The three-tier ranges are separate lists; the intermediate or narrow tiers of different parameters are never stated together ([0111], [0112]). | Support | 13, 16, 17, 35, 41–43, 67, 69, 70, 82 | medium | F-29-6 | Add sentences stating the intermediate tiers together and the narrow tiers together. | open |
| 33 | Same defect as row 9, limb on the range claims 16–17, 35, 41–43 and 68–70. | Support | 16, 17, 35, 41–43, 67–70, 76, 77, 82 | medium | F-30-20 | As row 9. | open |
| 34 | Same defect as row 9, stated for PEEK in claims 76–77. | Support | 16, 17, 35, 41–43, 67–70, 76, 77, 82 | medium | F-32-17 | As row 9. | open |
| 35 | In claim 31 supply air reaches the inner-surface gap only through the open main passage, so the purge path is not "separate from the main passage" as claim 1 requires. | Support | 1, 31 | medium | F-32-11 | Reword claim 31 so that the gap is reached through the open main passage and is distinct from it. | open |
| 36 | The package contradicts itself: [0140] says the ring is "not touched" below the threshold, while [0144] requires contact below it. | Support | 1, 40 | medium | F-36-15 | Amend [0140] to "not deflected so as to open the purge gap", with inventor confirmation; align claim 1 as in row 17. | open |
| 37 | Claimed features appear in no figure: integral flexure, return formation, inner-surface gap, openings in the wall, over-travel stop and depressor (37 CFR 1.83(a)). | Support | 24, 28, 31, 32, 45, 46 | medium | F-36-6 | Add schematic partial sections in the style of FIGS. 11–14. | open |
| 38 | (unclear) A ring that flexes axially, not radially, to open the purge path escapes every "radial deflection" claim; no such embodiment is disclosed. | Support | 1, 58, 67, 79 | medium | F-36-9 | If within the inventor's conception, disclose it and say the deformation may be radial or axial. | open |
| 39 | (unclear) No embodiment purges from first lift, with no normal stroke; every independent claim needs a normal-open state ([0073]). | Support | 1, 58, 79 | medium | F-36-10 | If conceived, disclose it and add an independent claim without the normal-open state; counsel weighs EP 1137892 A1 and US 8043186 B2. | open |
| 40 | "Inflatable chamber" is defined as bicycle-only ([0046]), so e-scooter, motorcycle and other sealant tyres are outside claims 58 and 79. | Support | 58, 79 | medium | F-36-4 | Redefine as any tyre or tube holding liquid sealant, with bicycles as the tested example. | open |
| 41 | Claim 64 claims the chamber as an article "comprising" a tube, while [0046] defines it as a volume. | Support | 64 | medium | F-37-3 | Align [0046] with the claim, or the claim with [0046]. | addressed in fix pass (not re-reviewed) |
| 42 | The same volume-versus-article conflict makes claim 64 open to an indefiniteness attack; [0052] equates the chamber with the tube. | Support | 64, 65 | medium | F-38-1 | Redefine [0046] as the physical article. | open |
| 43 | Claim 25 defines the product by a result on sealant film ("shears sealant film"), which is not part of the claim and is not measured. | Clarity | 25, 80 | medium | F-30-14 | Recast kinematically: the ring moves across and in contact with the poppet surface during recovery. | open |
| 44 | Same result-language problem; against the separate wipers of US 4719938 A the distinction is the recovering ring, which the claim should recite. | Clarity | 25, 80 | medium | F-32-19 | As row 43. | open |
| 45 | Clarity terms examiners will query: a "closed" gap with no criterion, "at-rest form", "low-adhesion", "tyre set-up", "surfaces on which sealant film accumulates". | Clarity | 1, 25, 44, 49, 55, 64, 79, 95 | medium | F-30-15 | Reformulate each from existing description text, without changing scope. | open |
| 46 | No generic definition of a "closed" purge gap (the "no significant flow" test in [0127] covers one location only), and "to its at-rest form" conflicts with admitted residual set ([0121]). | Clarity | 1, 58, 67, 79 | medium | F-36-11 | Add a generic "closed" definition to [0039] and a "towards the at-rest form" wording to [0034]. | open |
| 47 | [0030] defines the key terms as "any structure capable of performing the recited function ... and their equivalents", inviting US means-plus-function treatment that confines claim 1 to the described structures. | Clarity | 1, 2, 28, 36, 62 | medium | F-30-1 | Replace with structural definitions ([0035], [0042], [0043]) and state that means-plus-function treatment is not intended. | open |
| 48 | Same [0030] wording, applied to "deflecting formation" and "threshold-setting arrangement". | Clarity | 1, 2, 36 | medium | F-gate14-7 | As row 47; counsel to consider structural wording in claim 36. | open |
| 49 | "Threshold-setting arrangement" pairs a nonce word with a function and is a likely US means-plus-function term; "deflecting formation" is close to the line. | Clarity | 2, 36 | medium | F-37-11 | As row 47. | open |
| 50 | 102 claims, five independents, and multiple dependent claims resting on other multiple dependent claims, which the US forbids (37 CFR 1.75(c)); excess-claim fees in all three offices. | Formal | 1, 10, 18, 58, 61, 64, 66, 67, 89, 99 | medium | F-30-13 | Restructure into single dependencies for the US; justify or fold the product independents; prune to the intended ladder. | open |
| 51 | The multiple-on-multiple dependency defect runs through 56 claims (US only). | Formal | 10, 14, 18, 22–25, 27–29, 32–36, 40–47, 49–52, 54–57, 64, 66, 70–73, 75, 76, 78, 82–89, 93–96, 98–100, 102 | medium | F-37-1 | As row 50; no narrowing. | addressed in fix pass (not re-reviewed) |
| 52 | The claim structure is not adapted to US multiple-dependency rules or EPO Rule 43(2). | Formal | 1, 10, 13, 58, 61, 64, 67, 89, 99 | medium | F-gate14-3 | Counsel prepares a single-dependency US set and the EPO interrelated-products justification. | open |
| 53 | Required description sections are missing (title, field, background, problem, summary, brief description of drawings). | Formal | — | medium | F-30-21 | As row 11. | open |
| 54 | The figure plan carries internal references and statements of doubt ("not a confirmed mechanism", "the inventor states doubt") that must not reach the drawings. | Formal | 1, 21, 30 | medium | F-28-8 | Strip them; write the brief description of the drawings from the description. | open |
| 55 | Same drafting-process content, plus a FIG. 1 instruction that cannot be drawn as written. | Formal | — | medium | F-32-5 | Strip them; state one at-rest condition for FIG. 1. | open |
| 56 | The figure plan also has stale flags, and [0111] says dimensions are called out in FIG. 6A when the instructions call out none. | Formal | — | medium | F-gate14-8 | Add the call-outs to FIG. 6A; re-run the numeral cross-check; unify the numeral style. | open |
| 57 | Disclosed alternatives outside the claims (purge actuator [0145], depressor-only [0088], inverted ring [0139], deflation-only [0089]): US dedication risk and literal escape. | Design-around | 1, 46, 57, 58, 79 | medium | F-30-2 | As row 15. | open |
| 58 | A near-two-state design with the threshold close to the crack pressure ([0073]) sits at the edge of claim 1's normal-open state. | Design-around | 1, 58, 79 | medium | F-32-6 | Consider an independent claim with the closed and purge states only. | open |
| 59 | [0073] contemplates a purge on substantially every opening; the variant with no real normal-open state is outside every independent claim. | Design-around | 1, 58, 79 | medium | F-33-1 | Add an independent claim (core and method) with the closed and purge states only ([0035], [0073], [0087]). | open |
| 60 | The two-state, depressor-actuated and deflation-only modes ([0073], [0088], [0089]) have no independent claim. | Design-around | 1, 58, 79 | medium | F-36-16 | As row 15. | open |
| 61 | Over-travel by depressor, by hand, during deflation only, or by a separate actuator falls outside claims 1, 58 and 79. | Design-around | 1, 46, 58, 79 | medium | F-gate14-10 | As row 15; add a claim to the purge-actuator form ([0145]). | open |
| 62 | Claim 1 needs the ring "carried by the core body" and the poppet to cause the deflection; the purge actuator ([0145]) and the ring on the poppet ([0139]) escape. | Design-around | 1, 58 | medium | F-33-9 | Add wording in which any member moved by the pressure differential causes the deflection, and the ring may be on the core body or the poppet. | open |
| 63 | "Carried by", read with [0026] (indirect carriage), arguably takes in a ring on the poppet, which widens the art against claim 1; [0139] cuts the other way. Scope is uncertain either way. | Design-around | 1, 18, 24 | medium | F-32-9 | Recite the ring on one of core body and poppet and the deflecting formation on the other. | open |
| 64 | (unclear) A ring that flexes axially, or is pushed bodily off a land, escapes "radial deflection" ([0041]). | Design-around | 1, 58, 67, 79 | medium | F-33-13 | If conceived, disclose it and claim "elastic deformation" without "radial"; counsel checks US 4408632 A and US 8043186 B2 first. | open |
| 65 | A permanent small clearance that carries no real flow is never "closed", so it escapes claims 1, 58 and 79. | Design-around | 1, 58, 79 | medium | F-33-14 | As row 46. | open |
| 66 | Recovery "to" the at-rest form, and closing "below the over-travel threshold", are escaped by a ring with residual set ([0121]). | Design-around | 1, 58, 67, 79 | medium | F-33-17 | Recite recovery "towards" the at-rest form and closing "as the pressure differential falls". | open |
| 67 | Superelastic hysteresis can keep the gap open below the threshold, so the closing condition of claims 1 and 58 is not met literally ([0096]). | Design-around | 1, 40, 58 | medium | F-32-14 | Use claim 79's wording: the gap closes as the pressure differential falls. | open |
| 68 | The wipe-only embodiment ([0087]) has no claim family. | Design-around | 1, 25, 79, 80 | medium | F-36-3 | As row 18, with a matching method claim. | open |
| 69 | Claim 67 is limited to Presta-sized rings, so Schrader-scale rings ([0159]) and rings just outside the window escape. | Design-around | 67 | medium | F-33-6 | Consider a dimension-free ring claim, keeping claims 16, 17 and 78 as fallbacks; weigh against row 13. | open |
| 70 | Claim 67 is also escaped by a wall convoluted along its whole length, because [0049] measures diameter at a convolution-free portion. | Design-around | 67 | medium | F-33-15 | Add a fallback measurement convention to [0049], e.g. the diameter at the sealing edge. | open |
| 71 | A matched ring, poppet and spring kit sold without a core body infringes no product claim directly. | Design-around | 1, 67 | medium | F-33-18 | Add a kit claim ([0163], FIG. 5). | open |
| 72 | The kit has no claim and the component claim is dimension-bound. | Design-around | 1, 67 | medium | F-gate14-9 | As row 71. | open |
| 73 | No claims to non-bicycle sealant chambers, the kit, or the valve with a depressor head or adaptor ([0088], [0161], [0163]). | Design-around | 64, 65, 66 | medium | F-36-8 | Add a generic chamber claim, a kit claim and an inflation-system claim. | open |
| 74 | Claim 90 is performed in the supplier's factory and escaped by air cooling or by photochemical etching ([0173]). | Design-around | 90 | medium | F-33-11 | Move water quenching and laser cutting into dependent claims; counsel weighs EPO intermediate generalisation. | open |
| 75 | Method claim 79 is performed by the cyclist, and infringement depends on proving sealant film on the accused surfaces. | Design-around | 79, 81 | medium | F-33-12 | Recite "across a surface of the deflectable ring element" ([0078]); move the film wording to a dependent claim. | open |
| 76 | [0199] calls the stainless-steel core body a matter of general materials knowledge, which is a ready obviousness admission against claim 54. | Inventive step | 54 | low | F-38-3 | Reword [0199] to describe the corrosion consideration without the admission. | open |
| 77 | No design relationship links over-travel, convolution amplitude and strain; the key values were design values, not measurements ([0110], [0113]). | Enablement | 13, 35, 43 | low | F-30-17 | Add the relationship used, or state that the over-travel stop limits the ring load. | open |
| 78 | The Schrader variant has no operating sequence for a pin that every pump head presses ([0159]). | Enablement | 46, 57 | low | F-36-13 | Add the Schrader operating sequence and the depressor travel. | open |
| 79 | (unclear) Behaviour on top-up inflation from riding pressure is not described; every test ran from fully deflated. | Enablement | 1, 79, 87 | low | F-36-5 | Add after [0091] whether the purge state is reached on top-up, or say that it is unknown. | open |
| 80 | (unclear) Any test data beyond Example 1 (more cycles, cold, PEEK, wiper ring) is not in the text ([0182], [0188]). | Enablement | 1 | low | F-36-17 | Ask the inventor; add any results as actual examples. | open |
| 81 | "Defined wiping edge" is disclosed only as the laser-cut sharp edge ([0149]), yet claimed for any ring. | Support | 27 | low | F-28-14 | Define it in the definitions and extend it to other rings only if the inventor agrees. | open |
| 82 | Some claimed combinations are supported only implicitly, e.g. the 0.03 mm interference with a polymer ring. | Support | 9, 18, 20, 26, 59, 63, 102 | low | F-28-19 | Add one confirming sentence per combination, or prune. | open |
| 83 | Blanket statements ([0027], [0165]) will not secure an amendment basis at the EPO or in the UK. | Support | 1 | low | F-30-10 | Add explicit fallback paragraphs from existing content (as row 31). | open |
| 84 | No ranges for the normal stroke, the shoulder's rest clearance or the spring-rate ratio ([0067], [0043]). | Support | 2, 37, 41, 43 | low | F-36-12 | Ask the inventor for ranges they conceived; do not generate them. | open |
| 85 | No ranges for the normal stroke, the interference or the shape-setting window ([0170], [0195], [0198]). | Support | 20, 94, 101 | low | F-gate14-11 | As row 84. | open |
| 86 | (unclear) No narrower Af value to retreat to if cold performance becomes the battleground ([0095]). | Support | 48, 74 | low | F-36-19 | Add a nested ladder only if the inventor has chosen values. | open |
| 87 | (unclear) No basis to amend to recovery speed, force or the audible snap ([0099], [0181]). | Support | 1, 25, 47 | low | F-29-9 | Add a measured recovery time or force if one exists; otherwise rely on the material claims. | open |
| 88 | The diameter convention of [0049] fails for a ring convoluted end to end. | Support | 16, 67 | low | F-36-21 | As row 70. | open |
| 89 | [0100] ("no heating") reads against the thermally actuated purge of [0164]. | Support | — | low | F-38-2 | Confine [0100] to the over-travel embodiments. | open |
| 90 | The inverted ring and the purge actuator have no fallback claim ([0139], [0145]). | Support | 1, 58 | low | F-36-18 | As row 62. | open |
| 91 | (unclear) A spring-less poppet closed by chamber pressure is not disclosed. | Support | 1, 58, 79 | low | F-36-20 | As row 19. | open |
| 92 | "A purge gap associated with the deflectable ring element" states no physical relationship. | Clarity | 1, 58, 79 | low | F-30-19 | Replace with "bounded at least in part by, or formed through, the deflectable ring element" ([0124]–[0129]). | open |
| 93 | "About", "approximately" and "nominally" in claims, while [0028] says they define no tolerance. | Clarity | 20, 44, 49, 75, 98, 101 | low | F-30-23 | Recast claim 44 on the sub-ranges of [0112]; get tolerances from the inventor or supplier for the rest. | open |
| 94 | Claim 1's closing shape-recovery clause is not framed parallel to the three states. | Clarity | 1 | low | F-37-5 | Add a lead-in phrase. | open |
| 95 | "The chamber side" is applied to the fixed seat and the purge gap, but [0031] defines it only for the poppet. | Clarity | 9, 33 | low | F-37-10 | Broaden [0031] to a general orientation. | open |
| 96 | [0183] cross-refers to a background that is not in the package. | Formal | 1 | low | F-28-15 | Re-point [0183] to [0045] and [0050]. | open |
| 97 | Wording drift: "not exceeding" against "below" ([0064]); "to" against "towards" ([0034]); numeral 30 used for ring 730 in claim 58. | Formal | 1, 58 | low | F-28-20 | Align the wording and numerals. | open |
| 98 | Numeral 30 is attached to the genus in claims 1 and 58, and numeral 34 is reused for the fixed seat's sealing edge. | Formal | 1, 58 | low | F-30-25 | Drop or widen the numeral; give the fixed seat's edge its own numeral. | open |
| 99 | Claim 58 uses (30) for ring 730; claims 100–102 recite assembly under a ring-manufacturing preamble. | Formal | 58, 100–102 | low | F-30-26 | Claim 58 "(30, 730)"; retitle claims 100–102 as making a valve core. | open |
| 100 | Numeral format drift ([0051]–[0062] unbracketed), registry mismatches, and the claim 100 preamble. | Formal | 58, 100–102 | low | F-32-22 | Bracket numerals uniformly; correct the registry rows. | open |
| 101 | The genus "deflectable ring element (30)" carries the seat-ring numeral, although claim 9 makes the wiper ring (132) the element. | Formal | 1, 8, 9 | low | F-37-7 | Delete the numeral from the genus. | addressed in fix pass (not re-reviewed) |
| 102 | Claims 79–89 carry no reference numerals, unlike the other independents. | Formal | 79–89 | low | F-37-8 | Add numerals once the drawings exist. | open |
| 103 | Same reference-sign inconsistency across the claim set. | Formal | 79–89 | low | F-40a-3 | As row 102. | addressed in fix pass (not re-reviewed) |
| 104 | (unclear) The independent claims are not in EPO two-part form; whether that is "appropriate" depends on the closest art found by the search report. | Formal | 1, 58, 67, 79, 90 | low | F-40a-4 | Decide at EP and UK entry, after the search report. | open |
| 105 | No statement that reference signs in the claims are non-limiting. | Formal | — | low | F-40a-5 | Add the standard sentence at [0001]. | open |
| 106 | "Return spring" is the species; [0030] names the genus "biasing member". | Design-around | 1, 58, 79 | low | F-33-19 | Use "a biasing member (50)" in the independents ([0030], [0043]). | open |
| 107 | Method claim 79 is performed by the end user and is confined to bicycle chambers by [0046]. | Design-around | 79 | low | F-33-21 | Keep product claims free of in-use requirements; see row 40. | open |
| 108 | A depressor adaptor gives over-travel without "raising the pressure differential"; claim 79 is then not performed. | Design-around | 79 | low | F-35-6 | Add a method claim with over-travel by mechanical depression ([0088], [0089]). | open |
| 109 | US 9982818 B2 (UTC Aerospace Systems, 2018): a NiTi sealing ring with a wave-like section giving radial displacement. It is a static, thermally actuated seal, with no poppet, no sealing edge and no superelastic recovery. Closest art for the ring shape and material. | Prior art | 10–13, 47, 67–78, 90 | high | US 9982818 B2 | Rely on the moving-poppet seat, over-travel and purge; narrow claim 67 as in row 13. | prior art |
| 110 | US 6393765 B1 (US Navy, 2002): superelastic NiTi seal elements that deform on closure and recover when opened (hatches, doors). Shows superelastic NiTi was known as a repeatedly deforming seal. | Prior art | 47–50, 67, 73–75 | high | US 6393765 B1 | Combination partner for row 109; answer with the valve-seat role and the over-travel purge. | prior art |
| 111 | US 11904644 B2 (Santa Cruz Bicycles, 2024): a tubeless valve made because Presta and Schrader cores clog with sealant; a resilient seat with annular flow around it. No deforming seat, over-travel or wipe. Likely primary reference for the problem. | Prior art | 1, 58, 64–66, 79, 85 | medium | US 11904644 B2 | Distinguish on the deflecting ring, the separate purge path and recovery. | prior art |
| 112 | US 4719938 A (Helix Technology, 1988): wiper rings, pushed outward by a spring ring, wipe debris from the sealing surface as the valve closes. A combination partner against the wipe. | Prior art | 25–27, 80 | medium | US 4719938 A | Recite the wipe by recovery of the seat ring itself (row 43). | prior art |
| 113 | US 11345196 B2 (Kunshan Fuluo, 2022): tyre valve where a seal ring on a movable rod squeezes repair liquid off the wall when the rod is reinserted. Wiping against sealant, but by a maintenance movement. | Prior art | 25–27, 80 | medium | US 11345196 B2 | Distinguish on automatic wiping by seat recovery. | prior art |
| 114 | US 4462449 A (Zabel, 1984): tyre valve core in which soil is "swept through" on inflation and the core is "self cleaning". Early inflation-flow self-cleaning; no seat ring or wipe. | Prior art | 1, 79, 81 | medium | US 4462449 A | Distinguish on the pressure-threshold purge path opened by ring deflection. | prior art |
| 115 | US 8043186 B2 (Schaeffler, 2011): one check valve with two spring-set opening pressures, the second opening a second annular flow path. Paths are opened by separate members, not by a deflecting seat. A combination partner against the normal-stroke-then-over-travel sequence. | Prior art | 1, 36–39, 58, 79 | medium | US 8043186 B2 | Distinguish on one poppet deflecting a resilient ring that recloses by shape recovery. | prior art |
| 116 | EP 1137892 A1 (Sumnett, 2001): tyre valve core with a calibrated spring; above a set pressure the disk exposes bleed apertures, a second path in a tyre core opened above a threshold. Claim 1 not yet charted. | Prior art | 1, 36, 58 | medium | EP 1137892 A1 | Chart claim 1 from the saved text; distinguish on ring deflection and supply-to-chamber purge. | prior art |
| 117 | US 4408632 A (Brunswick, 1983): poppet with concentric primary and secondary seats opening paths in sequence as it travels. Background for a later-stage second path. | Prior art | 1 | low | US 4408632 A | Background only on the saved text. | prior art |
| 118 | US 4627461 A (K.J. Baillie, 1986): the closure member is rotated against the seat on closing to grind deposits away. An alternative seat-cleaning mechanism. | Prior art | 25–27 | low | US 4627461 A | Distinguish on recovery wiping without rotation. | prior art |
| 119 | US 6006954 A (Bespak, 1999): a wiper on a metering-valve stem scrapes deposits from the chamber wall on both strokes. | Prior art | 25–27, 80 | low | US 6006954 A | As row 112. | prior art |
| 120 | CN 215861914 U (Jinan Weidisi, 2022): exhaust air flow blows debris off a filter beside the sealing gasket in a tyre valve; read in machine translation. | Prior art | 79, 81, 88 | low | CN 215861914 U | Distinguish on the purge path through a deflected ring. | prior art |
| 121 | US 12215796 B2 (Santa Cruz Bicycles, 2025): an external seat and a resilient seating surface on a through-stem. No deforming seat, over-travel or purge. | Prior art | 1, 58 | low | US 12215796 B2 | Background; shows the field solves clogging by enlarging the seat. | prior art |
| 122 | US 11179979 B2 (Sport Components, 2021): a duckbill valve at the stem base that keeps sealant out. Exclusion, not removal. Claim 1 not yet charted. | Prior art | 58, 64, 65 | low | US 11179979 B2 | Background; chart claim 1 from the saved text. | prior art |
| 123 | US 11065925 B2 (Liquitube, 2021): clog-resistant stem with an extension tube and side air holes. Same problem, flow-path solution. | Prior art | 58, 64 | low | US 11065925 B2 | Background. | prior art |
| 124 | EP 4735276 A1 (applicant not identified, published 6 May 2026): replaces the Presta core with a ball valve to cure clogging. Published before any filing here, so it is full prior art. | Prior art | 1, 58 | low | EP 4735276 A1 | Background; take the applicant from the saved text. | prior art |
| 125 | US 9297373 B2 (Nuovo Pignone, 2016): poppet with two parallel non-linear springs. Background for a non-linear return spring; not sequential stages. | Prior art | 37 | low | US 9297373 B2 | Background. | prior art |
| 126 | Intake test document ref-01: sealant-resistant core with removable insert, low-adhesion poppet, rigid seat. The file states it and its number are fictional. | Prior art | 1, 55 | low | intake/prior-art/ref-01-sealant-resistant-core.md | Chart from the intake file only; do not cite a number. | prior art |
| 127 | Intake test document ref-02: elastomer lip seat that deflects outward under inflation pressure (main path only). The file states it and its number are fictional. | Prior art | 1, 8, 52 | low | intake/prior-art/ref-02-elastomer-seat-valve.md | Chart from the intake file only; distinguish on the separate purge path opened only in over-travel. | prior art |
| CN-1 | claims 4, 5 / purge shoulder (44) on the chamber side of the sealing face, bearing axially on the deflectable ring element in over-travel / the axial order of internal shoulder (24), sealing edge (34), convolutions (32a, 32b) and purge shoulder (44), and the ring surface the purge shoulder bears on, from the inventor's FIG-1 sketch; [0059], [0074], [0101], [0138] reconciled (F-28-3, F-32-20, F-36-1, F-gate14-2) | support (claims need disclosure) | 4 | medium | claims fix pass | add supporting disclosure before filing, or amend the claim | open |
| CN-2 | claim 22 / interference fit confined to an axial band adjacent the internal shoulder, remainder free of the core body bore / the route by which the purge flow in the outer-surface purge gap passes the interference band to reach the chamber side in the embodiment of FIGS. 1 to 4 (F-28-3, F-36-1) | support (claims need disclosure) | 22 | medium | claims fix pass | add supporting disclosure before filing, or amend the claim | open |
| CN-3 | claim 33 / purge gap bounded by the inner surface of the seat ring and the poppet on the chamber side of the sealing edge, flow along the poppet stem / placement of this gap consistent with [0110] (ring bore admits only the poppet stem) (F-28-3) | support (claims need disclosure) | 33 | medium | claims fix pass | add supporting disclosure before filing, or amend the claim | open |
| CN-4 | claims 30, 31, 32 / sense of radial deflection (outward or inward at the bounding surface) and the outer-surface purge gap / the prototype's actual kinematics: moving surface, sense, purge gap location; [0041], [0061], [0075], [0076] and the FIG. 1, 3, 6 instructions conformed, and the closed-state clearance removed from FIG. 1 (F-28-2, F-30-6, F-gate14-4) | support (claims need disclosure) | 30 | medium | claims fix pass | add supporting disclosure before filing, or amend the claim | open |
| CN-5 | claims 10, 84 / wiper ring deflected outward opening an outer-surface purge gap; seat ring deflecting radially outward / same kinematics confirmation for the wiper ring embodiment; [0115], [0147], [0185] and the FIG. 10B instruction conformed (F-28-2, F-gate14-4) | support (claims need disclosure) | 10 | medium | claims fix pass | add supporting disclosure before filing, or amend the claim | open |
| CN-6 | claims 1, 61, 69, 92 / normal-open state without the at-rest form of the deflectable ring element / sentence in [0140] and [0144] that the deflecting formation may contact and elastically load the ring below the over-travel threshold without opening the purge gap; 'is not touched' amended to 'is not deflected so as to open the purge gap'; inventor confirmation needed (F-33-2) | support (claims need disclosure) | 1 | medium | claims fix pass | add supporting disclosure before filing, or amend the claim | open |
| CN-7 | all claims / reference numerals (10, 12, 20, 22, 24, 30, 32, 34, 40, 41, 42, 44, 50, 52, 54, 60, 62, 64, 130, 132, 224, 232, 244, 254, 262, 324, 332, 344, 354, 362, 432, 444, 620, 630, 730) / drawing sheets, brief description of the drawings and abstract; without sheets the numerals point at nothing and figure-based support is at risk (F-gate14-1) | support (claims need disclosure) | see item | medium | claims fix pass | add supporting disclosure before filing, or amend the claim | open |

## How the score was reached

Novelty: 2 / 2. No saved reference discloses every feature of claim 1. The index finds nothing with a superelastic seat against which a moving member seals, nothing with the over-travel-plus-purge combination in any valve, and nothing with seat-recovery wiping. US 9982818 B2 does not anticipate claim 67 (static, thermally actuated seal; row 109).

Inventive step: 1 / 2. For claims 67 and 90 a credible combination exists (US 9982818 B2 with US 6393765 B1; rows 13–14). Claim 1 is arguable: its purge effect is not isolated by the data ([0182], [0190]; rows 20–21).

Support and enablement: 1 / 2. Open high findings and no critical one. The built embodiment's layout and purge mechanism are contradictory or missing (rows 1–8), and the ranges are tied to NiTi (row 9). Claim 1 survives on [0125]–[0128].

Clarity and form: 1 / 2. Open high formal findings (no drawings or abstract, unity; rows 10–12) and no critical one. The US dependency structure must be rebuilt (rows 50–52).

Eligibility and fallbacks: 1 / 2. A mechanical valve, eligible in the US, EP and GB. The claim ladder is deep (claims 2–57, 79–102), but at the EPO and UK IPO multi-list fallbacks rest on the blanket statement in [0165] (rows 31, 83), and the range fallbacks are tied to one material (row 9).

Per jurisdiction:

US: 7 / 10. Fallbacks score 2: the dependency defects are cured by preliminary amendment. Dedication of unclaimed alternatives (rows 15–18) costs scope, not grant.

EP: 6 / 10. Added-matter practice makes the missing layout unrepairable after filing and limits the fallback combinations; unity and Rule 43(2) apply to claims 1, 58, 67 and 90.

GB: 6 / 10. Added-matter practice (s.76) and breadth-of-claim sufficiency apply as at the EPO.

The score rests only on the 17 references we found and saved (coverage is thin: web-only, English-language, no CPC walk of F16K 15/20 or B60C 29/00). A professional search could change it. It is the application as reviewed in round 1; the fix pass running in parallel is not scored.

AI-assisted estimate for review by a qualified patent attorney or agent. Not legal advice and not a patentability opinion. Based only on the prior art identified in this matter.